ASSIGNMENT #2: Statistical Design of Experiments

Due electronically February 8th at midnight (11:59pm Toronto time)

Goal: Complete a Design of Experiment (DoE) and implement it on an Opentron-2 automated liquid handling platform equipped with a light sensor. Follow these steps:

- Using DoE, create an experiment that has comprehensive coverage of the search space for color mixing. Given an RYB combination, predict the expected color, and measure if the Opentron-2 platform creates the predicted color. The controllable experimental variables are

  - Three different colored solutions: red solution, yellow solution, and blue solution

  - Volume of solution(s) to be mixed

- Write python code to collect the data from the experiment performed by the Opentron-2

- Evaluate your DoE’s performance based on the collected data. Report the response surface for the data.

- Qualitatively identify and discuss any observed random and systemic errors in your collected data.

- Perform three-factor-factorial experiment ANOVA analysis for the collected data. Report all observed main factor effects and interaction effects.

- Describe the strategies you would implement to mitigate these the impact of these errors on your experimental results (think about it from experimental design perspective & data analysis perspective)

How to access the Opentron-2: The course TA will coordinate with you to obtain a one-hour time on the Opentron-2 platform. During your assigned time, you will connect to the Opentron-2 platform wirelessly using an API and your student ID.  There are two APIs available for use:

  - Single submission API via Hugging Face Interface (OPTIONAL): user interface to submit one color mixing task at a time by selecting a combination of dye volumes for red, yellow, and blue.

  - Batch submission API via Google Collab: python-based code templates to interact with OT2 API for a job submission with multiple measurements. You can use the provided scripts to submit tasks by uploading a CSV file containing your RYB library. Alternatively, you can modify the existing codes to design your own workflow for submitting your queries efficiently and storing the experimental results.

Submission instructions:

- Jupyter Notebook: Document the methods and processes of your DOE, including the experimental design and evaluation of the collected results with clear descriptions. (*You don’t need to upload the codes used to submit your tasks)

- CSV File: Attach a CSV file that records the input parameters you finally used (RYB combinations) along with their corresponding responses (the 8-channel light intensity values).

Information:

- To ensure reproducibility, set the random seed to 403 or 1003 for all tasks involving randomness

- Test Library Validity: Before submitting your final parameter sets, please use the provided scripts (in the colab) and id ‘debug’ to ensure the parameters you selected are valid.

- Input Parameter Ranges: For each dye, the volume can be varied from 1 µL to 299 µL. However, due to platform limitations, the error and variability increase significantly when the selected volume is less than 10 µL. Therefore, it is recommended to use volume values equal to or greater than 10 µL (e.g., 10, 11, 12, 13 µL, and so on) to ensure more accurate and reliable results.

- Submission Quota: Each of you will have a total quota of 26 submissions (25 + 1) for color mixing tasks using RYB combinations. The additional submission can be used to test the validity of your code and verify the mixing process (which is highly suggested) to ensure the platform functions as expected. Alternatively, you can include it as part of your main submissions.

- Equipment Time Allocation: (To Be Posted on Quercus via Announcement): Each student is allocated a specific one-hour time slot for submitting tasks and receiving responses. You will be able to access the OT-2 platform exclusively during your registered time slot by entering your student ID. Please ensure that:

  - Your code for job submission is prepared and ready to use before your time slot.

  - You have planned the RYB (Red, Yellow, Blue) combinations you intend to submit in advance.

  - If you require more than one slot (1 hour) for your DOE, please do not hesitate to reach out to me

- Results Collection: The OT-2 is equipped with a light sensor that measures the color intensity of the mixed solution at 8 different wavelengths. After passing the solution through the light sensor, the output will consist of 8 numerical values, each corresponding to a different wavelength channel.

- Job Processing: Each color mixing job will take approximately 2 minutes to complete. As shown in the Google Colab file, we’ve provided a function can sends an email notification with the results once the tasks are finished. You can modify the recipient’s email address to your own to avoid waiting for the results in real time.

- Similarity to real-world experiments: Uncontrollable effects or unmeasurable factors can influence the results. In color mixing tasks, the light sensor data is affected by ambient light from the OT-2, introducing approximately 10% noise in the responses. But fortunately, this noise is not random but consistent, and it is related to the position of the wells being used.

- Save your dataset. You will reuse it for Assignment #3.

- Since this assignment involves interaction with the SDL (OT-2), please do not hesitate to reach out to me if you have any questions. Additionally, if you would like to see the OT-2 platform in person, let me know—I can coordinate with our gracious host at the Acceleration Consortium to arrange a quick tour!

Notebook Contents: Your Jupyter notebook is a technical writing assignment as much as it is a coding assignment and science experiment. Emphasize conciseness and clear explanations in your report, along with visualizations and other details necessary for a reader to understand your thought process. In addition to the code, the notebook should have the following sections

- Introduction (markdown): Provide a concise overview of the strategy you used for your DOE and evaluation results of your collected data.

- Analysis: present analysis code and results, organized logically with markdown explanations.

- Summary: Key insights, conclusions, and reflections on your findings

Figures:

- DoE sampling strategy. Create a ternary diagram, where the three sides of the plot are red, yellow, and blue.

- Collected data. Create a ternary diagram response surface plot; the three sides of the plot are red, yellow, and blue.

- Comparison of the difference between the expected result and the experimental measurement

- If you have repeated measurements: Normal probability plot of residuals

Table: ANOVA analysis of your data

Evaluation Criteria

|  | Excellent (A+) | Good (A) | Acceptable (B) | Poor (C) | Unacceptable (D) |
| --- | --- | --- | --- | --- | --- |
| Coding | Strong organization indicating logic. Markdown shows separate major steps with detailed comments including relevant equations and theory. | Good organization, comments, code implements important methods but lacks some small details. Highly readable. | Some organization, comments indicate major sections but do not explain logical flow or important details | Little organization or logical flow. Few comments, or comments are AI generated. Missing results. | No organization or comments. Missing most results. |
| Correctness of Experiment | Methods indicate deep understanding of underlying assumptions and/or domain expertise. Cleverness and novelty. | Code can run with a single click and no errors. All methods implemented are correct for the task. Strong justification is provided for method choices. | Code runs but might use patches to accommodate a failed earlier step. A few poor method choices or weak justification. No random seed. | Some methods are inappropriate for the task, more than one method is incorrectly implemented. | Code does not run, or there are significant methodological errors including data leakage between train and test. |
| Analysis | Informed skepticism that identifies key assumptions and tests them. Work identifies novel research directions. | Formulates a valid scientific question. Correct conclusions based on results and correct reporting of implications for future work. | Comments on most results but misses a key aspect or assumption. Analysis does not consider implications for future work. | Comments miss most key results, do not comment on major trends. Conclusions are trivial. Conclusions do not reference results. | No analysis, incorrect conclusions, unfounded conclusions |